|  | Министерство науки и высшего образования Российской Федерации Федеральное государственное бюджетное образовательное учреждение высшего образования «Московский государственный технический университет имени Н.Э. Баумана (национальный исследовательский университет)» (МГТУ им. Н.Э. Баумана) |
| --- | --- |

ФАКУЛЬТЕТ Информатика и системы управления

КАФЕДРА ПРОГРАММНОЕ ОБЕСПЕЧЕНИЕ ЭВМ И ИНФОРМАЦИОННЫЕ ТЕХНОЛОГИИ (ИУ7)

НАПРАВЛЕНИЕ ПОДГОТОВКИ  09.03.04  Программная Инженерия

# Отчет

| по лабораторной работе № | 4 |
| --- | --- |

Название:

# Методологии разработки и верификации ускорителей вычислений на платформе XILINX ALVEO

Дисциплина: Архитектура ЭВМ

| Студент | ИУ7-51Б |  |  | Костев Д.И. |
| --- | --- | --- | --- | --- |
|  | (Группа) |  | (Подпись, дата) | (И.О. Фамилия) |
| Преподаватель |  |  |  | Попов А.Ю. |
|  |  |  | (Подпись, дата) | (И.О. Фамилия) |

Москва, 2021

Введение

Цель работы – изучение архитектуры гетерогенных вычислительных систем и технологии разработки ускорителей вычислений на базе ПЛИС фирмы Xilinx.

В ходе лабораторной работы предлагается изучить основные сведения о платформе Xilinx Alveo U200, разработать RTL (Register Transfer Language, язык регистровых передач)) описание ускорителя вычислений по индивидуальному варианту, выполнить генерацию ядра ускорителя, выполнить синтез и сборку бинарного модуля ускорителя, разработать и отладить тестирующее программное обеспечение на серверной хост-платформе, провести тесты работы ускорителя вычислений.

Номер варианта: 14, функция ускорителя:

R[i] = A[i]/16 - 11	SLR1,DDR[1]

Ход работы

На рисунке 1 представлена схема разрабатываемой аппаратной системы

Рисунок 1. Функциональная схема аппаратной системы

Последовательность событий транзакции чтения можно представить следующим образом: ARVALID→ ARREADY→ RVALID→ RREADY.

На рисунке 2 представлена диаграмма чтения данных.

Рисунок 2. Транзакция чтения

Последовательность событий транзакции записи: AWVALID→ AWREADY → WVALID → WREADY → BVALID → BREADY.

На рисунке3 представлена диаграмма записи данных.

Рисунок 3. Транзакция записи

На рисунке 4 представлена диаграмма инкремента данных в модуле rtl_kernel_wizard_0_adder.v

Рисунок 4. Диаграмма инкремента

На рисунке 5 представлено содержимое конфигурационного файла

Рисунок 5. Конфигурационный файл

Путь к конфиг. файлу: /iu_home/iu7134/workspace/ShatskiyR_lab1/rtl_kernel_wizard_1.cfg

Путь к .xo: /iu_home/iu7134/workspace/ShatskiyR_lab1_kernels/vivado_rtl_kernel/rtl_kernel_wizard_2_ex/exports/rtl_kernel_wizard_2.xo

Выходной файл: /iu_home/iu7134/workspace/vinc.xclbin_1

v++ -l -t hw -o /iu_home/iu7134/workspace/vinc.xclbin_1 -f xilinx_u200_xdma_201830_2 /iu_home/iu7134/workspace/ShatskiyR_lab1_kernels/vivado_rtl_kernel/rtl_kernel_wizard_2_ex/exports/rtl_kernel_wizard_2.xo --config /iu_home/iu7134/workspace/ShatskiyR_lab1/rtl_kernel_wizard_1.cfg

Содержимое файла v++_vinc.log представлено ниже:

INFO: [v++ 60-1306] Additional information associated with this v++ link can be found at:

Reports: /iu_home/iu7134/_x/reports/link

Log files: /iu_home/iu7134/_x/logs/link

INFO: [v++ 60-1548] Creating build summary session with primary output /iu_home/iu7134/workspace/vinc.xclbin_1.link_summary, at Tue Dec  7 00:09:26 2021

INFO: [v++ 60-1316] Initiating connection to rulecheck server, at Tue Dec  7 00:09:26 2021

INFO: [v++ 60-1315] Creating rulecheck session with output '/iu_home/iu7134/_x/reports/link/v++_link_vinc_guidance.html', at Tue Dec  7 00:09:45 2021

INFO: [v++ 60-895]   Target platform: /opt/xilinx/platforms/xilinx_u200_xdma_201830_2/xilinx_u200_xdma_201830_2.xpfm

INFO: [v++ 60-1578]   This platform contains Device Support Archive '/opt/xilinx/platforms/xilinx_u200_xdma_201830_2/hw/xilinx_u200_xdma_201830_2.dsa'

INFO: [v++ 74-74] Compiler Version string: 2020.2

INFO: [v++ 60-1302] Platform 'xilinx_u200_xdma_201830_2.xpfm' has been explicitly enabled for this release.

INFO: [v++ 60-629] Linking for hardware target

INFO: [v++ 60-423]   Target device: xilinx_u200_xdma_201830_2

INFO: [v++ 60-1332] Run 'run_link' status: Not started

INFO: [v++ 60-1443] [00:10:32] Run run_link: Step system_link: Started

INFO: [v++ 60-1453] Command Line: system_link --xo /iu_home/iu7134/workspace/ShatskiyR_lab1_kernels/vivado_rtl_kernel/rtl_kernel_wizard_1_ex/exports/rtl_kernel_wizard_1.xo --config /iu_home/iu7134/_x/link/int/syslinkConfig.ini --xpfm /opt/xilinx/platforms/xilinx_u200_xdma_201830_2/xilinx_u200_xdma_201830_2.xpfm --target hw --output_dir /iu_home/iu7134/_x/link/int --temp_dir /iu_home/iu7134/_x/link/sys_link

INFO: [v++ 60-1454] Run Directory: /iu_home/iu7134/_x/link/run_link

INFO: [SYSTEM_LINK 60-1316] Initiating connection to rulecheck server, at Tue Dec  7 00:10:43 2021

INFO: [SYSTEM_LINK 82-70] Extracting xo v3 file /iu_home/iu7134/workspace/ShatskiyR_lab1_kernels/vivado_rtl_kernel/rtl_kernel_wizard_1_ex/exports/rtl_kernel_wizard_1.xo

INFO: [SYSTEM_LINK 82-53] Creating IP database /iu_home/iu7134/_x/link/sys_link/_sysl/.cdb/xd_ip_db.xml

INFO: [SYSTEM_LINK 82-38] [00:10:45] build_xd_ip_db started: /data/Xilinx/Vitis/2020.2/bin/build_xd_ip_db -ip_search 0  -sds-pf /iu_home/iu7134/_x/link/sys_link/xilinx_u200_xdma_201830_2.hpfm -clkid 0 -ip /iu_home/iu7134/_x/link/sys_link/iprepo/mycompany_com_kernel_rtl_kernel_wizard_1_1_0,rtl_kernel_wizard_1 -o /iu_home/iu7134/_x/link/sys_link/_sysl/.cdb/xd_ip_db.xml

INFO: [SYSTEM_LINK 82-37] [00:11:16] build_xd_ip_db finished successfully

Time (s): cpu = 00:00:31 ; elapsed = 00:00:30 . Memory (MB): peak = 1557.891 ; gain = 0.000 ; free physical = 73951 ; free virtual = 243909

INFO: [SYSTEM_LINK 82-51] Create system connectivity graph

INFO: [SYSTEM_LINK 82-102] Applying explicit connections to the system connectivity graph: /iu_home/iu7134/_x/link/sys_link/cfgraph/cfgen_cfgraph.xml

INFO: [SYSTEM_LINK 82-38] [00:11:16] cfgen started: /data/Xilinx/Vitis/2020.2/bin/cfgen  -nk rtl_kernel_wizard_1:1:vinc0 -slr vinc0:SLR1 -sp vinc0.m00_axi:DDR[1] -dmclkid 0 -r /iu_home/iu7134/_x/link/sys_link/_sysl/.cdb/xd_ip_db.xml -o /iu_home/iu7134/_x/link/sys_link/cfgraph/cfgen_cfgraph.xml

INFO: [CFGEN 83-0] Kernel Specs:

INFO: [CFGEN 83-0]   kernel: rtl_kernel_wizard_1, num: 1  {vinc0}

INFO: [CFGEN 83-0] Port Specs:

INFO: [CFGEN 83-0]   kernel: vinc0, k_port: m00_axi, sptag: DDR[1]

INFO: [CFGEN 83-0] SLR Specs:

INFO: [CFGEN 83-0]   instance: vinc0, SLR: SLR1

INFO: [CFGEN 83-2228] Creating mapping for argument vinc0.axi00_ptr0 to DDR[1] for directive vinc0.m00_axi:DDR[1]

INFO: [SYSTEM_LINK 82-37] [00:11:42] cfgen finished successfully

Time (s): cpu = 00:00:25 ; elapsed = 00:00:26 . Memory (MB): peak = 1557.891 ; gain = 0.000 ; free physical = 73431 ; free virtual = 243399

INFO: [SYSTEM_LINK 82-52] Create top-level block diagram

INFO: [SYSTEM_LINK 82-38] [00:11:42] cf2bd started: /data/Xilinx/Vitis/2020.2/bin/cf2bd  --linux --trace_buffer 1024 --input_file /iu_home/iu7134/_x/link/sys_link/cfgraph/cfgen_cfgraph.xml --ip_db /iu_home/iu7134/_x/link/sys_link/_sysl/.cdb/xd_ip_db.xml --cf_name dr --working_dir /iu_home/iu7134/_x/link/sys_link/_sysl/.xsd --temp_dir /iu_home/iu7134/_x/link/sys_link --output_dir /iu_home/iu7134/_x/link/int --target_bd pfm_dynamic.bd

INFO: [CF2BD 82-31] Launching cf2xd: cf2xd -linux -trace-buffer 1024 -i /iu_home/iu7134/_x/link/sys_link/cfgraph/cfgen_cfgraph.xml -r /iu_home/iu7134/_x/link/sys_link/_sysl/.cdb/xd_ip_db.xml -o dr.xml

INFO: [CF2BD 82-28] cf2xd finished successfully

INFO: [CF2BD 82-31] Launching cf_xsd: cf_xsd -disable-address-gen -bd pfm_dynamic.bd -dn dr -dp /iu_home/iu7134/_x/link/sys_link/_sysl/.xsd

INFO: [CF2BD 82-28] cf_xsd finished successfully

INFO: [SYSTEM_LINK 82-37] [00:11:56] cf2bd finished successfully

Time (s): cpu = 00:00:12 ; elapsed = 00:00:14 . Memory (MB): peak = 1557.891 ; gain = 0.000 ; free physical = 73244 ; free virtual = 243314

INFO: [v++ 60-1441] [00:11:56] Run run_link: Step system_link: Completed

Time (s): cpu = 00:01:21 ; elapsed = 00:01:24 . Memory (MB): peak = 1576.969 ; gain = 0.000 ; free physical = 73282 ; free virtual = 243347

INFO: [v++ 60-1443] [00:11:56] Run run_link: Step cf2sw: Started

INFO: [v++ 60-1453] Command Line: cf2sw -sdsl /iu_home/iu7134/_x/link/int/sdsl.dat -rtd /iu_home/iu7134/_x/link/int/cf2sw.rtd -nofilter /iu_home/iu7134/_x/link/int/cf2sw_full.rtd -xclbin /iu_home/iu7134/_x/link/int/xclbin_orig.xml -o /iu_home/iu7134/_x/link/int/xclbin_orig.1.xml

INFO: [v++ 60-1454] Run Directory: /iu_home/iu7134/_x/link/run_link

INFO: [v++ 60-1441] [00:12:11] Run run_link: Step cf2sw: Completed

Time (s): cpu = 00:00:14 ; elapsed = 00:00:15 . Memory (MB): peak = 1576.969 ; gain = 0.000 ; free physical = 72433 ; free virtual = 242483

INFO: [v++ 60-1443] [00:12:11] Run run_link: Step rtd2_system_diagram: Started

INFO: [v++ 60-1453] Command Line: rtd2SystemDiagram

INFO: [v++ 60-1454] Run Directory: /iu_home/iu7134/_x/link/run_link

INFO: [v++ 60-1441] [00:12:21] Run run_link: Step rtd2_system_diagram: Completed

Time (s): cpu = 00:00:00 ; elapsed = 00:00:10 . Memory (MB): peak = 1576.969 ; gain = 0.000 ; free physical = 71874 ; free virtual = 241925

INFO: [v++ 60-1443] [00:12:21] Run run_link: Step vpl: Started

INFO: [v++ 60-1453] Command Line: vpl -t hw -f xilinx_u200_xdma_201830_2 --remote_ip_cache /iu_home/iu7134/.ipcache --output_dir /iu_home/iu7134/_x/link/int --log_dir /iu_home/iu7134/_x/logs/link --report_dir /iu_home/iu7134/_x/reports/link --config /iu_home/iu7134/_x/link/int/vplConfig.ini -k /iu_home/iu7134/_x/link/int/kernel_info.dat --webtalk_flag Vitis --temp_dir /iu_home/iu7134/_x/link --no-info --iprepo /iu_home/iu7134/_x/link/int/xo/ip_repo/mycompany_com_kernel_rtl_kernel_wizard_1_1_0 --messageDb /iu_home/iu7134/_x/link/run_link/vpl.pb /iu_home/iu7134/_x/link/int/dr.bd.tcl

INFO: [v++ 60-1454] Run Directory: /iu_home/iu7134/_x/link/run_link

****** vpl v2020.2 (64-bit)

**** SW Build (by xbuild) on 2020-11-18-05:13:29

** Copyright 1986-2020 Xilinx, Inc. All Rights Reserved.

INFO: [VPL 60-839] Read in kernel information from file '/iu_home/iu7134/_x/link/int/kernel_info.dat'.

INFO: [VPL 74-74] Compiler Version string: 2020.2

INFO: [VPL 60-423]   Target device: xilinx_u200_xdma_201830_2

INFO: [VPL 60-1032] Extracting hardware platform to /iu_home/iu7134/_x/link/vivado/vpl/.local/hw_platform

WARNING: /data/Xilinx/Vitis/2020.2/tps/lnx64/jre9.0.4 does not exist.

[00:17:46] Run vpl: Step create_project: RUNNING...

[00:17:38] Run vpl: Step create_project: Started

Creating Vivado project.

[00:18:07] Run vpl: Step create_project: Completed

[00:18:07] Run vpl: Step create_bd: Started

[00:19:51] Run vpl: Step create_bd: RUNNING...

[00:21:25] Run vpl: Step create_bd: RUNNING...

[00:22:55] Run vpl: Step create_bd: RUNNING...

[00:24:46] Run vpl: Step create_bd: RUNNING...

[00:26:18] Run vpl: Step create_bd: RUNNING...

[00:27:14] Run vpl: Step create_bd: Completed

[00:27:14] Run vpl: Step update_bd: Started

[00:27:17] Run vpl: Step update_bd: Completed

[00:27:17] Run vpl: Step generate_target: Started

[00:28:50] Run vpl: Step generate_target: RUNNING...

[00:30:25] Run vpl: Step generate_target: RUNNING...

[00:31:58] Run vpl: Step generate_target: RUNNING...

[00:33:40] Run vpl: Step generate_target: RUNNING...

[00:35:22] Run vpl: Step generate_target: RUNNING...

[00:36:13] Run vpl: Step generate_target: Completed

[00:36:13] Run vpl: Step config_hw_runs: Started

[00:36:26] Run vpl: Step config_hw_runs: Completed

[00:36:26] Run vpl: Step synth: Started

[00:38:26] Top-level synthesis in progress.

[00:39:03] Top-level synthesis in progress.

[00:39:40] Top-level synthesis in progress.

[00:40:16] Top-level synthesis in progress.

[00:40:56] Top-level synthesis in progress.

[00:41:36] Top-level synthesis in progress.

[00:42:14] Top-level synthesis in progress.

[00:42:57] Top-level synthesis in progress.

[00:43:36] Top-level synthesis in progress.

[00:44:16] Top-level synthesis in progress.

[00:44:55] Top-level synthesis in progress.

[00:45:37] Top-level synthesis in progress.

[00:46:15] Top-level synthesis in progress.

[00:46:53] Top-level synthesis in progress.

[00:47:31] Top-level synthesis in progress.

[00:48:12] Top-level synthesis in progress.

[00:49:02] Run vpl: Step synth: Completed

[00:49:02] Run vpl: Step impl: Started

[01:44:34] Finished 2nd of 6 tasks (FPGA linking synthesized kernels to platform). Elapsed time: 01h 32m 02s

[01:44:34] Starting logic optimization..

[01:53:17] Phase 1 Retarget

[01:55:09] Phase 2 Constant propagation

[01:57:07] Phase 3 Sweep

[02:01:24] Phase 4 BUFG optimization

[02:02:37] Phase 5 Shift Register Optimization

[02:03:13] Phase 6 Post Processing Netlist

[02:16:52] Finished 3rd of 6 tasks (FPGA logic optimization). Elapsed time: 00h 32m 17s

[02:16:52] Starting logic placement..

[02:21:15] Phase 1 Placer Initialization

[02:21:15] Phase 1.1 Placer Initialization Netlist Sorting

[02:35:03] Phase 1.2 IO Placement/ Clock Placement/ Build Placer Device

[02:43:26] Phase 1.3 Build Placer Netlist Model

[02:55:25] Phase 1.4 Constrain Clocks/Macros

[02:56:45] Phase 2 Global Placement

[02:56:45] Phase 2.1 Floorplanning

[02:59:52] Phase 2.1.1 Partition Driven Placement

[02:59:52] Phase 2.1.1.1 PBP: Partition Driven Placement

[03:01:07] Phase 2.1.1.2 PBP: Clock Region Placement

[03:04:57] Phase 2.1.1.3 PBP: Compute Congestion

[03:05:34] Phase 2.1.1.4 PBP: UpdateTiming

[03:07:26] Phase 2.1.1.5 PBP: Add part constraints

[03:08:03] Phase 2.2 Update Timing before SLR Path Opt

[03:08:41] Phase 2.3 Global Placement Core

[03:31:51] Phase 2.3.1 Physical Synthesis In Placer

[03:41:50] Phase 3 Detail Placement

[03:41:50] Phase 3.1 Commit Multi Column Macros

[03:42:28] Phase 3.2 Commit Most Macros & LUTRAMs

[03:47:31] Phase 3.3 Small Shape DP

[03:47:31] Phase 3.3.1 Small Shape Clustering

[03:49:32] Phase 3.3.2 Flow Legalize Slice Clusters

[03:49:32] Phase 3.3.3 Slice Area Swap

[03:54:06] Phase 3.4 Place Remaining

[03:54:06] Phase 3.5 Re-assign LUT pins

[03:55:23] Phase 3.6 Pipeline Register Optimization

[03:55:23] Phase 3.7 Fast Optimization

[03:59:43] Phase 4 Post Placement Optimization and Clean-Up

[03:59:43] Phase 4.1 Post Commit Optimization

[04:08:03] Phase 4.1.1 Post Placement Optimization

[04:08:40] Phase 4.1.1.1 BUFG Insertion

[04:08:40] Phase 1 Physical Synthesis Initialization

[04:11:12] Phase 4.1.1.2 BUFG Replication

[04:13:08] Phase 4.1.1.3 Replication

[04:18:56] Phase 4.2 Post Placement Cleanup

[04:19:34] Phase 4.3 Placer Reporting

[04:19:34] Phase 4.3.1 Print Estimated Congestion

[04:21:30] Phase 4.4 Final Placement Cleanup

[05:26:24] Finished 4th of 6 tasks (FPGA logic placement). Elapsed time: 03h 09m 32s

[05:26:24] Starting logic routing..

[05:31:23] Phase 1 Build RT Design

[05:40:53] Phase 2 Router Initialization

[05:40:53] Phase 2.1 Fix Topology Constraints

[05:41:29] Phase 2.2 Pre Route Cleanup

[05:42:06] Phase 2.3 Global Clock Net Routing

[05:43:58] Phase 2.4 Update Timing

[05:55:59] Phase 2.5 Update Timing for Bus Skew

[05:55:59] Phase 2.5.1 Update Timing

[06:00:21] Phase 3 Initial Routing

[06:00:21] Phase 3.1 Global Routing

[06:04:48] Phase 4 Rip-up And Reroute

[06:04:48] Phase 4.1 Global Iteration 0

[06:23:32] Phase 4.2 Global Iteration 1

[06:33:51] Phase 5 Delay and Skew Optimization

[06:33:51] Phase 5.1 Delay CleanUp

[06:33:51] Phase 5.1.1 Update Timing

[06:40:19] Phase 5.2 Clock Skew Optimization

[06:40:57] Phase 6 Post Hold Fix

[06:40:57] Phase 6.1 Hold Fix Iter

[06:40:57] Phase 6.1.1 Update Timing

[06:45:19] Phase 7 Route finalize

[06:45:57] Phase 8 Verifying routed nets

[06:47:11] Phase 9 Depositing Routes

[06:51:02] Phase 10 Route finalize

[06:51:02] Phase 11 Post Router Timing

[06:57:17] Finished 5th of 6 tasks (FPGA routing). Elapsed time: 01h 30m 52s

[06:57:17] Starting bitstream generation..

[08:49:26] Creating bitmap...

[09:36:16] Writing bitstream ./pfm_top_i_dynamic_region_my_rm_partial.bit...

[09:36:54] Finished 6th of 6 tasks (FPGA bitstream generation). Elapsed time: 02h 39m 37s

[09:39:53] Run vpl: Step impl: Completed

[09:40:01] Run vpl: FINISHED. Run Status: impl Complete!

INFO: [v++ 60-1441] [09:40:24] Run run_link: Step vpl: Completed

Time (s): cpu = 00:16:56 ; elapsed = 09:28:03 . Memory (MB): peak = 1576.969 ; gain = 0.000 ; free physical = 113718 ; free virtual = 278521

INFO: [v++ 60-1443] [09:40:24] Run run_link: Step rtdgen: Started

INFO: [v++ 60-1453] Command Line: rtdgen

INFO: [v++ 60-1454] Run Directory: /iu_home/iu7134/_x/link/run_link

INFO: [v++ 60-991] clock name 'clkwiz_kernel_clk_out1' (clock ID '0') is being mapped to clock name 'DATA_CLK' in the xclbin

INFO: [v++ 60-991] clock name 'clkwiz_kernel2_clk_out1' (clock ID '1') is being mapped to clock name 'KERNEL_CLK' in the xclbin

INFO: [v++ 60-1230] The compiler selected the following frequencies for the runtime controllable kernel clock(s) and scalable system clock(s): Kernel (DATA) clock: clkwiz_kernel_clk_out1 = 300, Kernel (KERNEL) clock: clkwiz_kernel2_clk_out1 = 500

INFO: [v++ 60-1453] Command Line: cf2sw -a /iu_home/iu7134/_x/link/int/address_map.xml -sdsl /iu_home/iu7134/_x/link/int/sdsl.dat -xclbin /iu_home/iu7134/_x/link/int/xclbin_orig.xml -rtd /iu_home/iu7134/_x/link/int/vinc.rtd -o /iu_home/iu7134/_x/link/int/vinc.xml

INFO: [v++ 60-1652] Cf2sw returned exit code: 0

INFO: [v++ 60-2311] HPISystemDiagram::writeSystemDiagramAfterRunningVivado, rtdInputFilePath: /iu_home/iu7134/_x/link/int/vinc.rtd

INFO: [v++ 60-2312] HPISystemDiagram::writeSystemDiagramAfterRunningVivado, systemDiagramOutputFilePath: /iu_home/iu7134/_x/link/int/systemDiagramModelSlrBaseAddress.json

INFO: [v++ 60-1618] Launching

INFO: [v++ 60-1441] [09:40:38] Run run_link: Step rtdgen: Completed

Time (s): cpu = 00:00:13 ; elapsed = 00:00:14 . Memory (MB): peak = 1576.969 ; gain = 0.000 ; free physical = 113641 ; free virtual = 278504

INFO: [v++ 60-1443] [09:40:38] Run run_link: Step xclbinutil: Started

INFO: [v++ 60-1453] Command Line: xclbinutil --add-section DEBUG_IP_LAYOUT:JSON:/iu_home/iu7134/_x/link/int/debug_ip_layout.rtd --add-section BITSTREAM:RAW:/iu_home/iu7134/_x/link/int/partial.bit --force --target hw --key-value SYS:dfx_enable:true --add-section :JSON:/iu_home/iu7134/_x/link/int/vinc.rtd --append-section :JSON:/iu_home/iu7134/_x/link/int/appendSection.rtd --add-section CLOCK_FREQ_TOPOLOGY:JSON:/iu_home/iu7134/_x/link/int/vinc_xml.rtd --add-section BUILD_METADATA:JSON:/iu_home/iu7134/_x/link/int/vinc_build.rtd --add-section EMBEDDED_METADATA:RAW:/iu_home/iu7134/_x/link/int/vinc.xml --add-section SYSTEM_METADATA:RAW:/iu_home/iu7134/_x/link/int/systemDiagramModelSlrBaseAddress.json --output /iu_home/iu7134/workspace/vinc.xclbin_1

INFO: [v++ 60-1454] Run Directory: /iu_home/iu7134/_x/link/run_link

XRT Build Version: 2.8.743 (2020.2)

Build Date: 2020-11-16 00:19:11

Hash ID: 77d5484b5c4daa691a7f78235053fb036829b1e9

Creating a default 'in-memory' xclbin image.

Section: 'DEBUG_IP_LAYOUT'(9) was successfully added.

Size   : 440 bytes

Format : JSON

File   : '/iu_home/iu7134/_x/link/int/debug_ip_layout.rtd'

Section: 'BITSTREAM'(0) was successfully added.

Size   : 39912114 bytes

Format : RAW

File   : '/iu_home/iu7134/_x/link/int/partial.bit'

Section: 'MEM_TOPOLOGY'(6) was successfully added.

Format : JSON

File   : 'mem_topology'

Section: 'IP_LAYOUT'(8) was successfully added.

Format : JSON

File   : 'ip_layout'

Section: 'CONNECTIVITY'(7) was successfully added.

Format : JSON

File   : 'connectivity'

Section: 'CLOCK_FREQ_TOPOLOGY'(11) was successfully added.

Size   : 274 bytes

Format : JSON

File   : '/iu_home/iu7134/_x/link/int/vinc_xml.rtd'

Section: 'BUILD_METADATA'(14) was successfully added.

Size   : 3089 bytes

Format : JSON

File   : '/iu_home/iu7134/_x/link/int/vinc_build.rtd'

Section: 'EMBEDDED_METADATA'(2) was successfully added.

Size   : 2759 bytes

Format : RAW

File   : '/iu_home/iu7134/_x/link/int/vinc.xml'

Section: 'SYSTEM_METADATA'(22) was successfully added.

Size   : 5793 bytes

Format : RAW

File   : '/iu_home/iu7134/_x/link/int/systemDiagramModelSlrBaseAddress.json'

Section: 'IP_LAYOUT'(8) was successfully appended to.

Format : JSON

File   : 'ip_layout'

Successfully wrote (39934570 bytes) to the output file: /iu_home/iu7134/workspace/vinc.xclbin_1

Leaving xclbinutil.

INFO: [v++ 60-1441] [09:40:40] Run run_link: Step xclbinutil: Completed

Time (s): cpu = 00:00:00.46 ; elapsed = 00:00:02 . Memory (MB): peak = 1576.969 ; gain = 0.000 ; free physical = 113575 ; free virtual = 278515

INFO: [v++ 60-1443] [09:40:40] Run run_link: Step xclbinutilinfo: Started

INFO: [v++ 60-1453] Command Line: xclbinutil --quiet --force --info /iu_home/iu7134/workspace/vinc.xclbin.info --input /iu_home/iu7134/workspace/vinc.xclbin_1

INFO: [v++ 60-1454] Run Directory: /iu_home/iu7134/_x/link/run_link

INFO: [v++ 60-1441] [09:40:43] Run run_link: Step xclbinutilinfo: Completed

Time (s): cpu = 00:00:02 ; elapsed = 00:00:03 . Memory (MB): peak = 1576.969 ; gain = 0.000 ; free physical = 113543 ; free virtual = 278482

INFO: [v++ 60-1443] [09:40:43] Run run_link: Step generate_sc_driver: Started

INFO: [v++ 60-1453] Command Line:

INFO: [v++ 60-1454] Run Directory: /iu_home/iu7134/_x/link/run_link

INFO: [v++ 60-1441] [09:40:43] Run run_link: Step generate_sc_driver: Completed

Time (s): cpu = 00:00:00.01 ; elapsed = 00:00:00.05 . Memory (MB): peak = 1576.969 ; gain = 0.000 ; free physical = 113545 ; free virtual = 278484

INFO: [v++ 60-244] Generating system estimate report...

INFO: [v++ 60-1092] Generated system estimate report: /iu_home/iu7134/_x/reports/link/system_estimate_vinc.xtxt

INFO: [v++ 60-586] Created /iu_home/iu7134/workspace/vinc.ltx

INFO: [v++ 60-586] Created /iu_home/iu7134/workspace/vinc.xclbin_1

INFO: [v++ 60-1307] Run completed. Additional information can be found in:

Guidance: /iu_home/iu7134/_x/reports/link/v++_link_vinc_guidance.html

Timing Report: /iu_home/iu7134/_x/reports/link/imp/impl_1_xilinx_u200_xdma_201830_2_bb_locked_timing_summary_routed.rpt

Vivado Log: /iu_home/iu7134/_x/logs/link/vivado.log

Steps Log File: /iu_home/iu7134/_x/logs/link/link.steps.log

INFO: [v++ 60-2343] Use the vitis_analyzer tool to visualize and navigate the relevant reports. Run the following command.

vitis_analyzer /iu_home/iu7134/workspace/vinc.xclbin_1.link_summary

INFO: [v++ 60-791] Total elapsed time: 9h 32m 7s

INFO: [v++ 60-1653] Closing dispatch client.

Содержимое файла vinc.xclbin.info приведено ниже:

==============================================================================

XRT Build Version: 2.8.743 (2020.2)

Build Date: 2020-11-16 00:19:11

Hash ID: 77d5484b5c4daa691a7f78235053fb036829b1e9

==============================================================================

xclbin Information

------------------

Generated by:           v++ (2020.2) on 2020-11-18-05:13:29

Version:                2.8.743

Kernels:                rtl_kernel_wizard_1

Signature:

Content:                Bitstream

UUID (xclbin):          f3f63637-0591-4499-be99-4754be03ad0e

Sections:               DEBUG_IP_LAYOUT, BITSTREAM, MEM_TOPOLOGY, IP_LAYOUT,

CONNECTIVITY, CLOCK_FREQ_TOPOLOGY, BUILD_METADATA,

EMBEDDED_METADATA, SYSTEM_METADATA,

GROUP_CONNECTIVITY, GROUP_TOPOLOGY

==============================================================================

Hardware Platform (Shell) Information

-------------------------------------

Vendor:                 xilinx

Board:                  u200

Name:                   xdma

Version:                201830.2

Generated Version:      Vivado 2018.3 (SW Build: 2568420)

Created:                Tue Jun 25 06:55:20 2019

FPGA Device:            xcu200

Board Vendor:           xilinx.com

Board Name:             xilinx.com:au200:1.0

Board Part:             xilinx.com:au200:part0:1.0

Platform VBNV:          xilinx_u200_xdma_201830_2

Static UUID:            c102e7af-b2b8-4381-992b-9a00cc3863eb

Feature ROM TimeStamp:  1561465320

Clocks

------

Name:      DATA_CLK

Index:     0

Type:      DATA

Frequency: 300 MHz

Name:      KERNEL_CLK

Index:     1

Type:      KERNEL

Frequency: 500 MHz

Memory Configuration

--------------------

Name:         bank0

Index:        0

Type:         MEM_DDR4

Base Address: 0x4000000000

Address Size: 0x400000000

Bank Used:    No

Name:         bank1

Index:        1

Type:         MEM_DDR4

Base Address: 0x5000000000

Address Size: 0x400000000

Bank Used:    Yes

Name:         bank2

Index:        2

Type:         MEM_DDR4

Base Address: 0x6000000000

Address Size: 0x400000000

Bank Used:    No

Name:         bank3

Index:        3

Type:         MEM_DDR4

Base Address: 0x7000000000

Address Size: 0x400000000

Bank Used:    No

Name:         PLRAM[0]

Index:        4

Type:         MEM_DRAM

Base Address: 0x3000000000

Address Size: 0x20000

Bank Used:    No

Name:         PLRAM[1]

Index:        5

Type:         MEM_DRAM

Base Address: 0x3000200000

Address Size: 0x20000

Bank Used:    No

Name:         PLRAM[2]

Index:        6

Type:         MEM_DRAM

Base Address: 0x3000400000

Address Size: 0x20000

Bank Used:    No

==============================================================================

Kernel: rtl_kernel_wizard_1

Definition

----------

Signature: rtl_kernel_wizard_1 (uint scalar00, int* axi00_ptr0)

Ports

-----

Port:          s_axi_control

Mode:          slave

Range (bytes): 0x1000

Data Width:    32 bits

Port Type:     addressable

Port:          m00_axi

Mode:          master

Range (bytes): 0xFFFFFFFFFFFFFFFF

Data Width:    512 bits

Port Type:     addressable

--------------------------

Instance:        vinc0

Base Address: 0x1800000

Argument:          scalar00

Register Offset:   0x010

Port:              s_axi_control

Memory:            <not applicable>

Argument:          axi00_ptr0

Register Offset:   0x018

Port:              m00_axi

Memory:            bank1 (MEM_DDR4)

==============================================================================

Generated By

------------

Command:       v++

Version:       2020.2 - 2020-11-18-05:13:29 (SW BUILD: 0)

Command Line:  v++ --config /iu_home/iu7134/workspace/ShatskiyR_lab1/rtl_kernel_wizard_1.cfg --connectivity.nk rtl_kernel_wizard_1:1:vinc0 --connectivity.slr vinc0:SLR1 --connectivity.sp vinc0.m00_axi:DDR[1] --input_files /iu_home/iu7134/workspace/ShatskiyR_lab1_kernels/vivado_rtl_kernel/rtl_kernel_wizard_1_ex/exports/rtl_kernel_wizard_1.xo --link --optimize 0 --output /iu_home/iu7134/workspace/vinc.xclbin_1 --platform xilinx_u200_xdma_201830_2 --report_level 0 --target hw --vivado.prop run.impl_1.STEPS.OPT_DESIGN.ARGS.DIRECTIVE=Explore --vivado.prop run.impl_1.STEPS.PLACE_DESIGN.ARGS.DIRECTIVE=Explore --vivado.prop run.impl_1.STEPS.PHYS_OPT_DESIGN.IS_ENABLED=true --vivado.prop run.impl_1.STEPS.PHYS_OPT_DESIGN.ARGS.DIRECTIVE=AggressiveExplore --vivado.prop run.impl_1.STEPS.ROUTE_DESIGN.ARGS.DIRECTIVE=Explore

Options:       --config /iu_home/iu7134/workspace/ShatskiyR_lab1/rtl_kernel_wizard_1.cfg

--connectivity.nk rtl_kernel_wizard_1:1:vinc0

--connectivity.slr vinc0:SLR1

--connectivity.sp vinc0.m00_axi:DDR[1]

--input_files /iu_home/iu7134/workspace/ShatskiyR_lab1_kernels/vivado_rtl_kernel/rtl_kernel_wizard_1_ex/exports/rtl_kernel_wizard_1.xo

--link

--optimize 0

--output /iu_home/iu7134/workspace/vinc.xclbin_1

--platform xilinx_u200_xdma_201830_2

--report_level 0

--target hw

--vivado.prop run.impl_1.STEPS.OPT_DESIGN.ARGS.DIRECTIVE=Explore

--vivado.prop run.impl_1.STEPS.PLACE_DESIGN.ARGS.DIRECTIVE=Explore

--vivado.prop run.impl_1.STEPS.PHYS_OPT_DESIGN.IS_ENABLED=true

--vivado.prop run.impl_1.STEPS.PHYS_OPT_DESIGN.ARGS.DIRECTIVE=AggressiveExplore

--vivado.prop run.impl_1.STEPS.ROUTE_DESIGN.ARGS.DIRECTIVE=Explore

==============================================================================

User Added Key Value Pairs

--------------------------

<empty>

Индивидуальное задание

Функция ядра: R[i] = A[i]/16 – 11, регион: SLR1,DDR[1].

Ниже приведен измененный код ядра:

// This is a generated file. Use and modify at your own risk.

////////////////////////////////////////////////////////////////////////////////

// Description: Pipelined adder.  This is an adder with pipelines before and

//   after the adder datapath.  The output is fed into a FIFO and prog_full is

//   used to signal ready.  This design allows for high Fmax.

// default_nettype of none prevents implicit wire declaration.

`default_nettype none

`timescale 1ps / 1ps

module rtl_kernel_wizard_1_example_adder #(

parameter integer C_AXIS_TDATA_WIDTH = 512, // Data width of both input and output data

parameter integer C_ADDER_BIT_WIDTH  = 32,

parameter integer C_NUM_CLOCKS       = 1

)

(

input wire  [C_ADDER_BIT_WIDTH-1:0]   ctrl_constant,

input wire                             s_axis_aclk,

input wire                             s_axis_areset,

input wire                             s_axis_tvalid,

output wire                            s_axis_tready,

input wire  [C_AXIS_TDATA_WIDTH-1:0]   s_axis_tdata,

input wire  [C_AXIS_TDATA_WIDTH/8-1:0] s_axis_tkeep,

input wire                             s_axis_tlast,

input wire                             m_axis_aclk,

output wire                            m_axis_tvalid,

input  wire                            m_axis_tready,

output wire [C_AXIS_TDATA_WIDTH-1:0]   m_axis_tdata,

output wire [C_AXIS_TDATA_WIDTH/8-1:0] m_axis_tkeep,

output wire                            m_axis_tlast

);

localparam integer LP_NUM_LOOPS = C_AXIS_TDATA_WIDTH/C_ADDER_BIT_WIDTH;

localparam         LP_CLOCKING_MODE = C_NUM_CLOCKS == 1 ? "common_clock" : "independent_clock";

/////////////////////////////////////////////////////////////////////////////

// Variables

/////////////////////////////////////////////////////////////////////////////

reg                              d1_tvalid = 1'b0;

reg                              d1_tready = 1'b0;

reg   [C_AXIS_TDATA_WIDTH-1:0]   d1_tdata;

reg   [C_AXIS_TDATA_WIDTH/8-1:0] d1_tkeep;

reg                              d1_tlast;

reg   [C_ADDER_BIT_WIDTH-1:0]    d1_constant;

integer i;

reg                              d2_tvalid = 1'b0;

reg   [C_AXIS_TDATA_WIDTH-1:0]   d2_tdata;

reg   [C_AXIS_TDATA_WIDTH/8-1:0] d2_tkeep;

reg                              d2_tlast;

wire  [C_AXIS_TDATA_WIDTH/8-1:0] d2_tstrb;

wire  [0:0]                      d2_tid;

wire  [0:0]                      d2_tdest;

wire  [0:0]                      d2_tuser;

wire                             prog_full_axis;

reg                              fifo_ready_r = 1'b0;

/////////////////////////////////////////////////////////////////////////////

// RTL Logic

/////////////////////////////////////////////////////////////////////////////

// Register s_axis_interface/inputs

always @(posedge s_axis_aclk) begin

d1_tvalid <= s_axis_tvalid;

d1_tready <= s_axis_tready;

d1_tdata  <= s_axis_tdata;

d1_tkeep  <= s_axis_tkeep;

d1_tlast  <= s_axis_tlast;

d1_constant <= ctrl_constant;

end

// Adder function

always @(posedge s_axis_aclk) begin

for (i = 0; i < LP_NUM_LOOPS; i = i + 1) begin

d2_tdata[i*C_ADDER_BIT_WIDTH+:C_ADDER_BIT_WIDTH] <= d1_tdata[C_ADDER_BIT_WIDTH*i+:C_ADDER_BIT_WIDTH] / 16 - 11;

end

end

// Register inputs to fifo

always @(posedge s_axis_aclk) begin

d2_tvalid <= d1_tvalid & d1_tready;

d2_tkeep  <= d1_tkeep;

d2_tlast  <= d1_tlast;

end

// Tie-off unused inputs to FIFO.

assign d2_tstrb = {C_AXIS_TDATA_WIDTH/8{1'b1}};

assign d2_tid   = 1'b0;

assign d2_tdest = 1'b0;

assign d2_tuser = 1'b0;

always @(posedge s_axis_aclk) begin

fifo_ready_r <= ~prog_full_axis;

end

assign s_axis_tready = fifo_ready_r;

xpm_fifo_axis #(

.CDC_SYNC_STAGES     ( 2                      ) , // DECIMAL

.CLOCKING_MODE       ( LP_CLOCKING_MODE       ) , // String

.ECC_MODE            ( "no_ecc"               ) , // String

.FIFO_DEPTH          ( 32                     ) , // DECIMAL

.FIFO_MEMORY_TYPE    ( "distributed"          ) , // String

.PACKET_FIFO         ( "false"                ) , // String

.PROG_EMPTY_THRESH   ( 5                      ) , // DECIMAL

.PROG_FULL_THRESH    ( 32-5                   ) , // DECIMAL

.RD_DATA_COUNT_WIDTH ( 6                      ) , // DECIMAL

.RELATED_CLOCKS      ( 0                      ) , // DECIMAL

.TDATA_WIDTH         ( C_AXIS_TDATA_WIDTH     ) , // DECIMAL

.TDEST_WIDTH         ( 1                      ) , // DECIMAL

.TID_WIDTH           ( 1                      ) , // DECIMAL

.TUSER_WIDTH         ( 1                      ) , // DECIMAL

.USE_ADV_FEATURES    ( "1002"                 ) , // String: Only use prog_full

.WR_DATA_COUNT_WIDTH ( 6                      )   // DECIMAL

)

inst_xpm_fifo_axis (

.s_aclk             ( s_axis_aclk    ) ,

.s_aresetn          ( ~s_axis_areset ) ,

.s_axis_tvalid      ( d2_tvalid      ) ,

.s_axis_tready      (                ) ,

.s_axis_tdata       ( d2_tdata       ) ,

.s_axis_tstrb       ( d2_tstrb       ) ,

.s_axis_tkeep       ( d2_tkeep       ) ,

.s_axis_tlast       ( d2_tlast       ) ,

.s_axis_tid         ( d2_tid         ) ,

.s_axis_tdest       ( d2_tdest       ) ,

.s_axis_tuser       ( d2_tuser       ) ,

.almost_full_axis   (                ) ,

.prog_full_axis     ( prog_full_axis ) ,

.wr_data_count_axis (                ) ,

.injectdbiterr_axis ( 1'b0           ) ,

.injectsbiterr_axis ( 1'b0           ) ,

.m_aclk             ( m_axis_aclk   ) ,

.m_axis_tvalid      ( m_axis_tvalid ) ,

.m_axis_tready      ( m_axis_tready ) ,

.m_axis_tdata       ( m_axis_tdata  ) ,

.m_axis_tstrb       (               ) ,

.m_axis_tkeep       ( m_axis_tkeep  ) ,

.m_axis_tlast       ( m_axis_tlast  ) ,

.m_axis_tid         (               ) ,

.m_axis_tdest       (               ) ,

.m_axis_tuser       (               ) ,

.almost_empty_axis  (               ) ,

.prog_empty_axis    (               ) ,

.rd_data_count_axis (               ) ,

.sbiterr_axis       (               ) ,

.dbiterr_axis       (               )

);

endmodule

`default_nettype wire

Последовательность событий транзакции чтения можно представить следующим образом: ARVALID→ ARREADY→ RVALID→ RREADY.

На рисунке 6 представлена диаграмма чтения данных.

Рисунок 6. Транзакция чтения

Последовательность событий транзакции записи: AWVALID→ AWREADY → WVALID → WREADY → BVALID → BREADY.

На рисунке 7 представлена диаграмма записи данных.

Рисунок 7. Транзакция записи

На рисунке 8 представлена диаграмма инкремента данных в модуле rtl_kernel_wizard_0_adder.v

Рисунок 8. Диаграмма «adder’а»

В файле host_example.cpp были изменена часть кода с проверкой считанного результата операции:

for (cl_uint i = 0; i < number_of_words; i++) {

if ((h_data[i] / 16 - 11) != h_axi00_ptr0_output[i]) {

printf("ERROR in rtl_kernel_wizard_1::m00_axi - array index %d (host addr 0x%03x) - input=%d (0x%x), output=%d (0x%x)\n", i, i*4, h_data[i], h_data[i], h_axi00_ptr0_output[i], h_axi00_ptr0_output[i]);

check_status = 1;

}

//  printf("i=%d, input=%d, output=%d\n", i,  h_axi00_ptr0_input[i], h_axi00_ptr0_output[i]);

}

На рисунке 9 представлен результат проверки работоспособности программы.

Рисунок 9. Результат проверки

Контрольные вопросы

- Назовите преимущества и недостатки XDMA и QDMA платформ.

Сборка QDMA (Queue Direct Memory Access), доступная на картах ускорителей Alveo, предоставляет разработчикам прямое потоковое соединение с низкой задержкой между хостом и ядрами. Оболочка QDMA включает высокопроизводительный DMA, который использует несколько очередей, оптимизированных как для передачи данных с высокой пропускной способностью, так и для передачи данных с большим количеством пакетов. Только QDMA позволяет передавать поток данных непосредственно в логику FPGA параллельно с их обработкой.

Оболочка XDMA требует, чтобы данные сначала были полностью перемещены из памяти хоста в память FPGA (DDRx4 DIMM или PLRAM), прежде чем логика FPGA сможет начать обработку данных, что влияет на задержку на зап уска задачи.

Потоковая передача напрямую в работающие ускорительные ядра (так называемый Free-Running-Mode) позволяет быстро и без излишней буферизации передавать операнды и результаты вычислений на хост по потоковому интерфейсу AXI4 Stream. Решение QDMA подходит для приложений, в которых вычисления строятся на передачи сравнительно небольших пакетов, но при этом требуется высокая производительность и минимальная задержка отклика.

QDMA подходит для потоковой передачи небольших данных, а в остальных случаях больше подходит XDMA.

- Назовите последовательность действий, необходимых для инициализации ускорителя со стороны хост-системы.

- Получить название целевого ускорителя;

- Получить список всех доступных платформ;

- Выбрать вендора ускорителя (Xilinx);

- Получить список доступных вычислительных устройств;

- Выбрать целевого ускорителя.

- Какова процедура запуска задания на исполнения в ускорительном ядре VINC.

- Инициализация ускорителя;

- Создание вычислительного контекста целевого ускорителя;

- Создание очереди команд к ускорителю;

- Загрузка слинкованного из объектных файлов ядра бинарного файла в оперативную память (*.xclbin).

- Создание программы на основе загруженного бинарного файла.

- Настройка маппинга оперативной памяти на входы ядра.

- Запуск задания для ядра на подготовленных входных данных.

- Считывание выходных данных из устройства для проверки корректности работы ядра.

- Освобождение захваченных хост-программой ресурсов.

- Опишите процесс линковки на основании содержимого файла v++_*.log.

- SYSTEM_LINK

- CFGEN

- CF2BD

- Synthesis (block-level, top-level)

- FPGA linking synthesized kernels to platform

- FPGA logic optimization

- FPGA logic placement

- FPGA routing

- FPGA bitstream generation

- rtdgen

- xclbinutil

- xclbinutilinfo

Заключение

В ходе работы была изучена архитектура гетерогенных вычислительных систем и технологии разработки ускорителей вычислений на базе ПЛИС фирмы Xilinx.